AXI4-Stream Video BFM - SRED Information

Project Name:

SIMU / AXI4-Stream Video Source and Sink BFMs

Author and date:

Jacob von Chorus / June 26, 2018 / Project Completion

The purpose of this project was to implement a video source and sink BFM for providing input to and reading output from a video system. AXI4-Stream interfaces were used with the data format described in Xilinx’s AXI4-Stream Video IP and System Design Guide (UG934). Use of a standardized data format permitted the BMFs to be compatible with any video system compliant with Xilinx’s guideline.

The source BFM was required to generate video frames with colour spaces of: RGB444, YCC444, YCC422, and YCC420. Bit depth, resolution, and blanking were configurable along with the number of pixels transmitted per clock cycle.

The sink BFM received frames from its interface and determined their resolution. Frames could be compared to transmitted frames for errors and written to an image file for visual inspection.

Class based SystemVerilog was used for implementation to facilitate integration into testbenches.

What technical uncertainty are you trying to overcome?

The primary technical challenge was designing the system to be generic in terms of colour space, subsampling, test pattern generation, and frame checking.

- The BFMs had to store frame data in RGB and YCC with 444, 422, and 420 subsampling. They were also required to output these frames to image files for viewing.

- The source had to generate test frames with patterns that allowed for individual pixel values to be easily verified in a simulator, and for the entire frame to be checked visually.

- The sink had to determine frame dimensions automatically from the received stream.

- The BFMs required the ability to compare frames for differences.

- The BFMs were required to work with a configurable component bit depth.

What activities did you do to overcome the uncertainties identified above?

- Developed two test pattern generators.

- Created functions to read and write raw frames to file.

- Wrote logic to transmit video frames over AXI4-Stream, including the ability to send sub-sampled data in Xilinx’s AXI4-Stream Video format.

- Created a testbench connecting the source to sink BFM to test the functionality of each.

- Developed a utility to convert a raw YCC frame to a raw RGB frame.

Were you able to overcome the uncertainties?  Describe the technological advancement you achieved.

- Internally, frames were stored without subsampling, three components per pixel were stored covering both RGB and YCC. Subsampled frames had their redundant chroma values copied from the previous pixel, ie. the stored frame was up-sampled for storage, and down-sampled for transmission.

- Images were written to file as raw pixel data, which was converted to tiff images using the raw2tiff utility. A utility was written to convert YCC to RGB.

- Two different test frames could be generated: The first incremented each component of consecutive pixels by a constant so that pixel values could be easily verified within a simulator. The second was a colour bar pattern, useful for visual inspection.

  - Further flexibility was provided by allowing raw frames to be read from file, permitting any user-defined frame to be used.

- The sink determined frame width and height by counting the number of pixels between end of line signals, and by counting the number of lines between start of frame signals.

- The BFM was able to compare two frames bit-by-bit for differences.

- A SystemVerilog parameter stored bit depth to make this configurable within the source and sink.