# SIMU UVM capability

## SIMU code updates to handle UVM files inside the testbench

To test an RTL module, we can build a UVM testbench around it and run a UVM test to verify it. This testbench usually includes the following:

- some agents’ packages (agent, monitor, driver, scoreboard and etc.),

- some sequences to drive the transaction over the interfaces,

- some interface definitions,

- the environment where all these different agents and components are instantiated,

- the testbench which has the DUT and connections to the interfaces and environment,

- and the UVM testcase where we run sequences in different scenarios to test the DUT.

When there are some or all of the above UVM libraries available, we can include them in our SIMU environment. Here, the goal is to use -use_uvm switch when running a testcase in the SIMU to make sure all uvm files are compiled and added to the testbench and simulation.

There are a few steps needed to manually configure and prepare the testcase and they are explained in the project settings section.

In this section, the changes applied to the SIMU code to recognize the uvm files, compile them and utilize them in the simulation are explained.

SIMU code was updated to recognize -use_uvm switch and also be able to compile files in a file list given by -f filelist.txt in the compile_all.tcl file.

The following is the list of the changes:

- Using -use_uvm switch as the command line argument when running a testcase in SIMU will set a TCL variable called ::CMD_ARG_UVM. Eg:

SIMU_SHELL> run_testcase ../testcases/tc_XXX/tc_XXX.tcl -use_uvm

- CMD_ARG_UVM is declared and initialized to 0 in the scripts_config/config_cmd_line_options_default.tcl file.

- scripts_config/scripts_lib/simu.tcl (line 565) was updated to set the ::CMD_ARG_UVM to 1, if -use_uvm is used in the command line.

- scripts_config/tccommon_lib/tccommon_print_config_options.tcl (line 33) was updated to print config setting about using uvm.

- scripts_config/scripts_lib/utils.tcl was updated for simu_vlog command to recognize -f switch to compile filelists (lines 997 and line 1026).

- Changed compiling testcase command to pass the “define UVM_TESTBENCH” to it in the following file: scripts_config/config_settings_vsim.tcl (line 80).

- Modelsim.ini in the SIMU was updated to use new UVM version:

mtiUvm = $MODEL_TECH/../uvm-1.2

- Inside the project folder, the compile_all.tcl is updated to compile UVM files and pass define UVM_TESTBENCH macro to  tb file.

## Project settings

The following steps are in addition to the SIMU project settings mentioned in the confluence webpage: https://fidussys.atlassian.net/wiki/spaces/FPGARIPL/pages/2407333904/Adding+SIMU+to+your+project

### Copying all UVM files to the testcase folder

All of the UVM related files including agent package, sequences, environment, testbench and testcase should be copied in the uvm folder in the testcase directory.

### Updating tc_XXX.tcl file

The uvm folder path should be set inside the tc_XXX.tcl file as an environment variable “TCUVMDIR” which is used in the filelist for compilation:

set ::env(TCUVMDIR) $TESTCASESDIR/$TCFILENAME/uvm

### creating uvm_filelist.txt

All the needed UVM files should be listed in a uvm_filelist.txt as shown below:

# Agents

${TCUVMDIR}/i2c_agent/sources/i2c_agent_pkg.sv

# IFs

${TCUVMDIR}/i2c_agent/interfaces/i2c_if.sv

### updating compile_all.tcl file, updating tb.sv and testcase tc_XXX.sv files

The compile_all.tcl is updated in the previous step and it has the template, just make sure it is up to date. If not, follow the next step:

In the case of using -use_uvm switch, the UVM files should be compiled and testcase and tb files should be compiled by passing the UVM_TESTBENCH compiler directive. All of these should be guarded if CMD_ARG_UVM is set. The compile_all.tcl should be updated to compile uvm_filelist.txt as below:

if {$::CMD_ARG_UVM > 0} {

simu_vlog -sv -incr -timescale $TCTIMESCALE -cover -f "$TCSUBDIR/uvm_filelist.txt"

}

The tb.sv file compilation should be replaced by the following few lines:

if {$::CMD_ARG_UVM > 0} {

simu_vlog -sv -incr -d UVM_TESTBENCH -timescale $TCTIMESCALE "$TCSUBDIR/tb.sv"

} else {

simu_vlog -sv -incr -timescale $TCTIMESCALE "$TCSUBDIR/tb.sv"

}

Inside the tb.sv file, all the uvm related changes should be added inside `ifdef UVM_TESTBENCH .. `endif

Block.

The compilation of the testcase file is already considering the -use_uvm switch and is done in the previous section.

Tb.sv and testcase tc_XXX.sv files are compiled with compiler directive macro `define UVM_TESTBENCH passed through command line to the simu_vlog command, so those files may need to be modified with `ifdef UVM_TESTBENCH and `else to properly run the testcase with and without uvm files.

If you want to use the SystemVerilog testcase and use SIMU sim management package, you can add your own test code inside the testcase module. That should be added inside `ifdef UVM_TESTBENCH .. `endif

Block.

## Adding UVM agent and sequence to the test

You can use only a UVM agent package and needed sequences in the SIMU SV testcase. In this scenario, you need to instantiate all needed components and sequences inside the testcase module and run sequences there.

## I2c agent test example

A folder was created to test i2cSlave module using UVM i2c agent package. It is located under testcases_envFidus_uvm/tc_fidus_i2c_slave_uvm_agent.

### tc_fidus_i2c_slave_uvm_agent

In this test case, we drive interface to the i2cSlave module by using the i2c_agent driver and sequences.

The uvm_filelist.txt includes only i2c_agen_pkg and i2c_if files to compile and use in the test case. The tb.sv file doesn’t need to be modified. The tc_fidus_i2c_slave_uvm_agent.sv file is modified and the original part of the testcase module is put inside the `else .. `endif block and the new test case is added in the `ifdef UVM_TESTBENCH..`else block. In the new testcase the agent along with its config, driver and sequencer is created and assigned to the agent fields and the connection between driver and sequencer ports are done manually. Then the sequences are created and started. The test writes a few bytes into i2cSlave module and reads them back and compare them, if they match it would pass.

Screenshot of running the uvm testcase:

Screenshot of compiling uvm files:

Screenshot of the test transcript: